PHASE-1 RTL Design

AXI4-Lite is a simplified, non-bursting subset of the AMBA AXI protocol designed specifically for simple control-and-status-register communication. It eliminates the complex burst tracking, out-of-order execution, and data interleaving of full AXI, making it ideal for the memory-mapped slave you are verifying.

The protocol separates reads and writes into five unidirectional, independent channels:

- Write Address (AW): Master sends the target memory address and control signals.

- Write Data (W): Master sends the actual data and byte strobes (WSTRB).

The Write Strobe (WSTRB) signal acts as a byte-enable mask. It tells the slave exactly which bytes within the 32-bit (or 64-bit) data bus actually contain valid data that should be written to the register or memory.

Because AXI4-Lite only supports full data-bus-width transfers, the master uses WSTRB to perform smaller, partial-word writes (like updating a single 8-bit byte inside a 32-bit register) without overwriting the adjacent bytes.

### How It Works

There is exactly one WSTRB bit for every 8 bits (one byte) of WDATA. For a standard 32-bit AXI4-Lite bus, WDATA is 32 bits wide, so WSTRB is 4 bits wide.

- WSTRB[0] controls WDATA[7:0] (Byte 0 - Lowest byte)

- WSTRB[1] controls WDATA[15:8] (Byte 1)

- WSTRB[2] controls WDATA[23:16] (Byte 2)

- WSTRB[3] controls WDATA[31:24] (Byte 3 - Highest byte)

If WSTRB[n] is 1, the slave must write that specific byte into the register. If WSTRB[n] is 0, the slave must ignore that byte and leave the existing register value intact.

### Common Examples (32-bit Bus)

- WSTRB = 4'b1111: A full 32-bit word write. All 4 bytes are updated.

- WSTRB = 4'b0001: A single byte write. Only the lowest 8 bits are updated. The upper 24 bits of the register remain unchanged.

- WSTRB = 4'b0011: A half-word (16-bit) write. The lower 16 bits are updated.

- WSTRB = 4'b0000: A null write. The master is sending data, but nothing should be written to the register.

### Why This is Critical for Your UVM Verification

If your slave ignores the WSTRB signal and blindly writes all 32 bits of WDATA whenever a write occurs, it is a critical protocol violation.

In your UVM environment, you must verify this behavior:

- Sequences: Your constrained-random sequences should generate writes with randomized WSTRB values (e.g., partial writes, sparse strobes like 4'b1010, and all-zero strobes).

- RTL Implementation: Your SystemVerilog slave logic needs a byte-by-byte write enable condition. (e.g., if (WSTRB[0]) reg_data[7:0] <= WDATA[7:0];).

- Scoreboard: Your UVM scoreboard must use the WSTRB value from the monitor to accurately predict the expected state of the register. It must apply a bitwise mask to combine the old register value with the new WDATA based on the strobe bits.

- Write Response (B): Slave sends a 2-bit status response indicating write success or failure.

- Read Address (AR): Master sends the target memory address and control signals.

- Read Data (R): Slave sends the requested data and a 2-bit status response.

One correction to keep in mind from the formal AXI specification: each channel has its own independent VALID/READY handshake, and a transfer occurs on a clock edge when both are high. AXI4-Lite also has no bursts; each access is a single transfer.

The important rule is:

Once RVALID is asserted, the slave cannot simply withdraw the response because RREADY is low.

This is exactly the sort of behavior our assertions and backpressure tests will check. The same VALID-until-READY principle applies to the other channels.

It has five independent channels:

### Write

1. Write Address Channel

AWADDR

AWVALID

AWREADY

2. Write Data Channel

WDATA

WSTRB

WVALID

WREADY

3. Write Response Channel

BRESP

BVALID

BREADY

### Read

4. Read Address Channel

ARADDR

ARVALID

ARREADY

5. Read Data Channel

RDATA

RRESP

RVALID

RREADY

# The really important case: AW first, W later

AXI allows the address and data channels to operate independently.

So this is legal:

Cycle 1:

AWVALID = 1

AWREADY = 1

→ address accepted

Cycle 2:

WVALID = 1

WREADY = 1

→ data accepted

Internally, our slave needs to remember:

aw_received = 1

and store:

awaddr_reg = AWADDR

Then when W arrives:

w_received = 1

Now:

aw_received && w_received

becomes true.

Only then should the register actually be updated.

This is one reason our RTL needs some internal state.

# 8. The opposite: W first, AW later

This is equally important.

Cycle 1:

WVALID = 1

WREADY = 1

→ data accepted

Cycle 2:

AWVALID = 1

AWREADY = 1

→ address accepted

Now the slave has:

stored_data    = WDATA

stored_strobes = WSTRB

stored_address = AWADDR

Once both are available:

address_received && data_received

we perform the write.

This is a very important verification scenario.

A naive AXI implementation can accidentally assume:

AW arrives first

and fail when W arrives first.

Our constrained-random UVM environment should specifically find this kind of bug.

┌────────────────┐

AW channel ─────►│ Address        │

│ holding        │

│ register       │

└───────┬────────┘

│

│

▼

┌─────────────┐

│             │

│ Write       │

│ Controller  │

│             │

└──────┬──────┘

│

▲

│

┌───────┴────────┐

W channel ──────►│ Data / Strobe  │

│ holding        │

│ registers      │

└────────────────┘

VALID/READY Handshaking (Critical for Assertions & Backpressure): Every channel uses a strict two-way handshake. The source drives the payload and asserts VALID. The destination asserts READY when it can accept the payload. The transfer occurs on the rising clock edge when both are HIGH.

- Backpressure: Your slave tests backpressure by driving READY LOW, forcing the master to wait.

- Assertion Rules:

  - A source must not wait for READY before asserting VALID.

  - Once VALID is asserted, it must remain HIGH (along with the data/address payload) until the handshake completes. The source cannot change its mind and drop the transaction.

  - A destination is permitted to wait for VALID before asserting READY.

Addressing and Data Rules (Critical for Sequence Constraints)

- Single Transfers Only: Every transaction is a single transfer. Burst length is always 1.

- Bus Width: The data bus (WDATA, RDATA) must be exactly 32-bit or 64-bit.

- Alignment: All accesses must be aligned to the width of the data bus. An unaligned address is a protocol violation.

- Byte Strobes (WSTRB): Masters use strobes to indicate which specific bytes within the data word are valid. Your slave must respect these strobes to avoid corrupting adjacent bytes in a register.

Responses (Critical for Error Handling and Invalid Addresses) Your slave must return a 2-bit response for every transaction via BRESP (for writes) or RRESP (for reads).

- OKAY (2'b00): Normal access success.

- EXOKAY (2'b01): Exclusive access success (Not supported in AXI4-Lite; usually falls back to OKAY).

- SLVERR (2'b10): Slave Error. Your slave returns this when the master attempts to access an invalid or unmapped address, or attempts an unauthorized write (e.g., writing to a read-only register).

- DECERR (2'b11): Decode Error. Typically generated by the system interconnect when no slave exists at the requested address.

Reset Behavior (Critical for Reset Sequences) The protocol relies on an active-low synchronous reset (ARESETn).

- During reset, all VALID signals across all channels must be driven strictly LOW.

- READY signals can be driven to any value during reset. It is a common design practice to assert slave READY signals HIGH during reset so the slave can accept a transaction on the very first cycle after reset deasserts.

In digital design, backpressure is a flow-control mechanism where a receiving component (the slave) signals a sending component (the master) to temporarily stop sending data because it cannot process it fast enough.

It is the hardware equivalent of saying, "Hold on, my hands are full—wait a second before handing me the next item."

Here is how it works and why it matters in your AXI4-Lite project:

### The Problem It Solves

Imagine a high-speed processor (Master) trying to write configuration data to a slower peripheral device (Slave). If the Master just blasts data continuously, the Slave's internal buffers will overflow, and data will be lost.

### How Backpressure Works in AXI4-Lite

In AXI, backpressure is implemented using the READY signal.

- The Master puts data on the bus and asserts VALID = 1 (saying: "I have valid data for you").

- The Slave might be busy doing something else. It asserts READY = 0 (saying: "I am not ready to accept it yet").

- This READY = 0 is the backpressure. It "pushes back" against the flow of data.

- Because of the AXI protocol rules, the Master is now trapped: it must hold the data steady and keep VALID = 1 indefinitely until the Slave finally asserts READY = 1.

AXI4-Lite is a simplified AXI interface intended primarily for control/status register access.

Why are we deliberately testing backpressure?

This is one of the important parts of the project.

A beginner test might do:

VALID = 1

READY = 1

→ transaction

and everything works.

But real hardware cannot assume the other side is always ready.

For example:

Master                     Slave

AWVALID ────────────────►

AWREADY ◄────── 0 ──────

WAIT

AWVALID ────────────────►

AWREADY ◄────── 1 ──────

↑

transfer

Our verification environment should intentionally manipulate READY signals so that the DUT has to wait.

That allows us to verify that:

- data isn't lost

- addresses aren't lost

- VALID remains asserted correctly

- responses aren't lost

- transactions don't complete prematurely

- the slave recovers correctly once READY becomes asserted

This is much more meaningful than simply testing a few reads and writes.

AXI4-LITE SLAVE

│

┌──────────────┴──────────────┐

│                             │

▼                             ▼

WRITE PATH                      READ PATH

│                             │

AW capture                    AR capture

│                             │

W capture                         │

│                             │

▼                             ▼

Both available?                Decode address

│                             │

▼                             ▼

Address decode                 Generate RDATA

│                       Generate RRESP

▼                             │

Register write                        ▼

│                           RVALID

▼                             │

Generate BRESP                       │

│                             │

▼                             ▼

BVALID                        RREADY

The UVM environment will generate transactions such as:

axi_transaction

{

operation = WRITE;

address   = 0x08;

data      = 0x12345678;

}

The sequence generates these transactions.

The sequencer delivers them to the driver.

The driver converts the transaction into actual AXI signals.

For example:

axi_transaction

│

▼

Driver

│

▼

AWADDR   = 0x08

WDATA    = 0x12345678

AWVALID  = 1

WVALID   = 1

The DUT processes the signals.

Then the monitor observes what actually happened.

The scoreboard compares:

Expected behavior

VS

Observed behavior

Instead of manually writing:

write(0x00, 1);

write(0x04, 2);

read(0x00);

read(0x04);

we'll create randomized transactions.

Something conceptually like:

WRITE 0x00 → 0xA13F...

READ  0x08

WRITE 0x0C → 0x91...

READ  0x00

WRITE invalid address

READ  invalid address

...

And constrain the randomization so we generate meaningful combinations.

For example:

70% valid addresses

20% invalid addresses

10% boundary addresses

The exact distributions can be decided later.

TEST

│

▼

┌───────────────┐

│ UVM Test      │

└───────┬───────┘

│

▼

┌───────────────┐

│   Sequences   │

└───────┬───────┘

│

▼

┌───────────────┐

│   Sequencer   │

└───────┬───────┘

│

▼

┌───────────────┐

│    Driver     │

└───────┬───────┘

│

│ AXI4-Lite

▼

┌────────────────────────┐

│          DUT           │

│                        │

│   AXI4-Lite Slave      │

│          │             │

│          ▼             │

│    Register Bank       │

└──────────┬─────────────┘

│

▼

┌───────────────┐

│    Monitor    │

└───────┬───────┘

│

┌───────────┴───────────┐

▼                       ▼

┌─────────────┐         ┌─────────────┐

│ Scoreboard  │         │  Coverage   │

└─────────────┘         └─────────────┘

+ SystemVerilog Assertions

+ Randomized Regression

+ Functional Coverage

+ Code Coverage

We'll build it progressively:

### Phase 1 — Understand AXI4-Lite

We'll establish exactly how:

- AW channel works

- W channel works

- B channel works

- AR channel works

- R channel works

- VALID/READY works

- independent channels work

- backpressure works

### Phase 2 — Define our peripheral

We'll decide:

Data width      = 32-bit

Address width   = 32-bit

Register count  = 4–8

and define the register map.

### Phase 3 — Build the RTL

First get:

RESET

↓

READ

↓

WRITE

↓

VALID/READY

↓

BACKPRESSURE

↓

ERROR RESPONSES

working.

### Phase 4 — Basic simulation

Before UVM, we'll directly verify the RTL with a simple SystemVerilog testbench.

This is important because if something fails later, we'll know whether the problem is:

DUT

or

UVM environment

### Phase 5 — Build UVM

Then:

transaction

interface

sequence

sequencer

driver

monitor

agent

scoreboard

environment

test

### Phase 6 — Add constrained-random testing

Generate randomized:

- addresses

- read/write operations

- data

- stalls

- transaction ordering

### Phase 7 — Add assertions

Protocol properties such as:

VALID must remain asserted until READY

and other AXI correctness rules.

### Phase 8 — Add coverage

Functional coverage + cross coverage.

### Phase 9 — Regression

Run many randomized tests/seeds and collect:

Tests

↓

Pass/Fail

↓

Coverage

↓

Coverage gaps

↓

New constraints/tests

↓

Regression

### Phase 10 — Coverage convergence

The final goal isn't simply:

"The tests passed."

It is:

Functional correctness + protocol correctness + meaningful coverage + no unexplained coverage gaps.

When reset is asserted:

RESET

↓

CONTROL  = 0

STATUS   = 1

DATA_IN  = 0

DATA_OUT = 0

CONFIG   = 0

COUNT    = 0

VERSION  = 0x00010000

SCRATCH  = 0

And the AXI interface returns to an idle state.

We will later test reset:

- Before any transaction

- Between transactions

- During a pending read

- During a pending write

The exact behavior for reset during an active transaction will be specified carefully when we write the protocol assertions.

AXI4-Lite DUT

│

│

┌─────┴─────┐

│ Simulation │

└─────┬─────┘

│

┌─────────┴─────────┐

│                   │

Functional           Protocol

Verification         Assertions

│                   │

└─────────┬─────────┘

│

Regression

│

┌─────────┴─────────┐

│                   │

Functional          Code Coverage

Coverage

│                   │

└─────────┬─────────┘

▼

Coverage Analysis

│

▼

Verification Gaps

│

▼

New Tests / Constraints

│

▼

Regression

FSMs associated with it

One important clarification first: AXI4-Lite does not prescribe a particular FSM implementation. We are designing our own internal FSMs to manage the five independent AXI channels correctly.

For our project, I recommend separate read and write control FSMs, rather than one giant FSM.

There is actually another perfectly valid way to implement this.

Instead of:

WRITE_IDLE

WAIT_W

WAIT_AW

WRITE_EXEC

B_RESPONSE

we could use flags:

aw_received

w_received

and a smaller FSM:

WRITE_IDLE

WRITE_EXEC

B_RESPONSE

For example:

if (AWVALID && AWREADY)

aw_received <= 1;

if (WVALID && WREADY)

w_received <= 1;

if (aw_received && w_received)

execute_write();

This is often cleaner for AXI-Lite because the channels are independent.

For our project, I prefer the flag-based approach internally, while thinking about it as the four conditions:

00 → nothing

10 → AW only

01 → W only

11 → both

It will make the RTL less unnecessarily state-heavy and give us a good opportunity to discuss FSM vs control-flag architecture in an interview.

┌─────────────┐

│             │

│  READ_IDLE  │

│             │

└──────┬──────┘

│

ARVALID

& ARREADY

│

▼

┌─────────────┐

│             │

│ READ_RESP   │

│             │

└──────┬──────┘

│

RVALID

& RREADY

│

▼

READ_IDLE

AXI4-LITE SLAVE

│

┌─────────────────────┴─────────────────────┐

│                                           │

▼                                           ▼

WRITE CONTROL                               READ CONTROL

│                                           │

AW capture                                  AR capture

W capture                                       │

│                                           │

▼                                           ▼

AW/W tracking                              Address decode

│                                           │

▼                                           ▼

Write decode                                Read data mux

│                                           │

▼                                           ▼

Register bank                                RDATA/RRESP

│                                           │

▼                                           ▼

BRESP/BVALID                                RVALID

│                                           │

▼                                           ▼

BREADY                                     RREADY

And underneath:

REGISTER BANK

│

┌─────────────────┼─────────────────┐

▼                 ▼                 ▼

CONTROL            STATUS            DATA_IN

│                 │                 │

▼                 ▼                 ▼

DATA_OUT           CONFIG            COUNT

│                 │                 │

▼                 ▼                 ▼

VERSION           SCRATCH

# One subtle issue: when should READY be asserted?

This is something we should decide before coding.

A simple implementation could say:

AWREADY = 1 whenever we can accept an address

WREADY  = 1 whenever we can accept data

ARREADY = 1 whenever read path is idle

But there are tradeoffs.

For example, suppose we've already captured:

AWADDR

but haven't received W yet.

Should:

AWREADY

remain high?

No.

Otherwise the master could send another address and we'd have nowhere to store it.

Similarly:

AW captured

→ AWREADY = 0

W captured

→ WREADY = 0

until the current transaction has completed.

This means our slave is effectively designed to support one outstanding write transaction and one outstanding read transaction at a time.

That's perfectly reasonable for this project and keeps the RTL manageable.

# 21. The architecture we're choosing

So let's lock this in:

### Write path

One outstanding write

AW → holding register

W  → holding register

AW_received

W_received

when both:

execute write

generate B response

hold BVALID until BREADY

### Read path

One outstanding read

AR → holding register

decode

generate RDATA/RRESP

hold RVALID until RREADY

### Register bank

Address decoder

│

├── RW registers

├── RO registers

└── Invalid

IMP EXAMPLE

# Transaction we're trying to perform

Let's say the master wants:

WRITE 0x08 = 0xDEADBEEF

Recall:

0x08 = DATA_IN

So the final expected result is:

DATA_IN = 0xDEADBEEF

BRESP   = OKAY

The master sends:

AWADDR = 0x08

WDATA  = 0xDEADBEEF

WSTRB  = 4'b1111

But deliberately, W arrives before AW.

# 2. Internal registers

Our slave needs to remember what it has received.

We'll have:

awaddr_reg

wdata_reg

wstrb_reg

aw_received

w_received

Think of these as our slave's temporary storage.

Initially:

awaddr_reg = X

wdata_reg  = X

wstrb_reg  = X

aw_received = 0

w_received  = 0

And we're in:

WRITE_IDLE

# 3. Cycle 0 — Initial state

Before anything happens:

WRITE_IDLE

aw_received = 0

w_received  = 0

AWREADY = 1

WREADY  = 1

BVALID  = 0

The slave is basically saying:

"I have nothing pending. Give me an address or data."

# 4. Cycle 1 — W arrives first

The master presents:

WVALID = 1

WDATA  = DEADBEEF

WSTRB  = 1111

But:

AWVALID = 0

The slave has:

WREADY = 1

Therefore:

WVALID && WREADY

↓

1

So the W transaction occurs on the rising edge.

### Before edge

WVALID = 1

WREADY = 1

WDATA  = DEADBEEF

### At edge

W transfer

### After edge

Our internal state becomes:

wdata_reg     = DEADBEEF

wstrb_reg     = 1111

w_received    = 1

aw_received   = 0

So:

WRITE SIDE

Address:      NOT RECEIVED

Data:         RECEIVED ✓

wdata_reg = DEADBEEF

The controller is now effectively in:

WAIT_FOR_AW

# 5. What happens to WREADY now?

This is important.

We've already captured the write data.

We don't have another storage slot.

Therefore:

WREADY = 0

We don't want the master sending another W transaction.

So:

Before W

─────────

WREADY = 1

After W

───────

WREADY = 0

The slave is saying:

"I already have the data for the current transaction. I can't accept another one yet."

# 6. Cycle 2 — AW arrives

Now the master presents:

AWVALID = 1

AWADDR  = 0x08

Our slave has:

AWREADY = 1

Therefore:

AWVALID && AWREADY

↓

1

The address transfers.

At the rising edge:

awaddr_reg  <= 0x08

aw_received <= 1

Now our internal state is:

aw_received = 1

w_received  = 1

awaddr_reg  = 0x08

wdata_reg   = DEADBEEF

wstrb_reg   = 1111

We now have everything required.

# 7. The important condition

Our control logic effectively sees:

aw_received && w_received

which is:

1 && 1

↓

1

Therefore:

The write transaction is complete from the input-channel perspective.

Now we can execute it.

# 8. Cycle 3 — Write execution

We have:

Address = 0x08

Data    = DEADBEEF

WSTRB   = 1111

Decode:

0x08

↓

DATA_IN

Access type:

DATA_IN = RW

Therefore:

DATA_IN <= DEADBEEF

Because:

WSTRB = 1111

all four bytes are enabled.

So:

DATA_IN

↓

0xDEADBEEF

At the same time, we determine:

BRESP = OKAY

# 9. Now we enter the response state

The slave asserts:

BVALID = 1

BRESP  = OKAY

The key thing is:

The register write has already happened.

The B channel is simply telling the master:

"Your write has completed successfully."

So our conceptual state becomes:

B_RESPONSE

# 10. But now the master applies backpressure

Here's the interesting part.

The master says:

BREADY = 0

So:

BVALID = 1

BREADY = 0

Therefore:

BVALID && BREADY = 0

No response transfer occurs.

# 11. Cycle 4

Our slave must keep:

BVALID = 1

BRESP  = OKAY

So:

Cycle 4

BVALID ─────────────── 1

BREADY ─────────────── 0

BRESP  ───────────── OKAY

The FSM remains:

B_RESPONSE

# 12. Cycle 5

The master still isn't ready:

BREADY = 0

Again:

BVALID = 1

Nothing happens.

The slave continues waiting.

Cycle 5

BVALID = 1

BREADY = 0

STATE = B_RESPONSE

# 13. Cycle 6 — Master finally becomes ready

Now:

BREADY = 1

while:

BVALID = 1

Therefore:

BVALID && BREADY

↓

1

The response transfers.

TRANSFER

↓

BVALID = 1 ──────────●

BREADY = 1 ──────────●

BRESP  = OKAY

The write transaction is now finished.

# 14. Cycle 7 — Return to idle

Now we can clear our internal tracking:

aw_received <= 0

w_received  <= 0

and return to:

WRITE_IDLE

Then:

AWREADY = 1

WREADY  = 1

The slave can accept a new write.

# 15. Entire transaction in one table

Here's the complete sequence:

| Cycle | Event | aw_received | w_received | State |
| --- | --- | --- | --- | --- |
| 0 | Idle | 0 | 0 | WRITE_IDLE |
| 1 | W handshake | 0 | 1 | WAIT_AW |
| 2 | AW handshake | 1 | 1 | WRITE_EXEC |
| 3 | Register updated | 1 | 1 | B_RESPONSE |
| 4 | B stalled | 1 | 1 | B_RESPONSE |
| 5 | B stalled | 1 | 1 | B_RESPONSE |
| 6 | B handshake | 1 | 1 | B_RESPONSE |
| 7 | Cleanup | 0 | 0 | WRITE_IDLE |

And the important values:

Cycle 1:

WDATA = DEADBEEF

↓

wdata_reg = DEADBEEF

Cycle 2:

AWADDR = 08

↓

awaddr_reg = 08

Cycle 3:

08 → DATA_IN

↓

DATA_IN = DEADBEEF

↓

BRESP = OKAY

BVALID = 1

Cycle 4-5:

BREADY = 0

↓

BVALID remains 1

Cycle 6:

BREADY = 1

↓

B handshake

Cycle 7:

Return to IDLE

# 16. Now let's look at the actual signal timeline

This is the most useful picture:

C0      C1      C2      C3      C4      C5      C6      C7

│       │       │       │       │       │       │       │

CLK     ↑       ↑       ↑       ↑       ↑       ↑       ↑       ↑

│       │       │       │       │       │       │       │

WVALID          ███████

WREADY          ███████

WDATA           DEADBEEF

↑

W

handshake

AWVALID                  ███████

AWREADY                  ███████

AWADDR                   0x08

↑

AW

handshake

BVALID                           █████████████████████

BREADY                           ────────────────██████

BRESP                            OKAY

↑

B handshake

DATA_IN                          DEADBEEF

↑

register

update

This one diagram captures almost the entire write-control problem.

# 17. Now let's ask: what if W never arrives?

Suppose:

AW handshake

happens, but then the master takes 10 cycles before presenting W.

Our slave does:

aw_received = 1

w_received  = 0

and waits.

It must not:

perform the write

generate BVALID

lose the address

accept another AW

So:

AWREADY = 0

WREADY  = 1

BVALID  = 0

until W arrives.

That's another important verification scenario.

# 18. What if AW never arrives?

Same logic, reversed.

After W:

aw_received = 0

w_received  = 1

Then:

AWREADY = 1

WREADY  = 0

BVALID  = 0

We wait.

This is why the two holding registers + two flags architecture works nicely.

# 19. Now the really important insight

The FSM isn't really "AW state → W state."

Because AXI allows both orders.

The real state of the write controller is determined by:

AW       W

│        │

▼        ▼

received? received?

│        │

└────┬───┘

│

▼

┌───────────────┐

│               │

│  00  10  01   │

│      11       │

│               │

└───────────────┘

You can think of it as a 2-bit state machine:

aw_received w_received

00 → nothing received

10 → address received

01 → data received

11 → both received → execute

This is actually a very clean way to understand AXI's independent channels.

# 20. Why this is useful for UVM

Now imagine we're writing our randomized sequence.

We can explicitly generate:

Scenario 1:

AW + W same cycle

Scenario 2:

AW → wait 1 cycle → W

Scenario 3:

AW → wait 5 cycles → W

Scenario 4:

W → wait 1 cycle → AW

Scenario 5:

W → wait 5 cycles → AW

Then add:

BREADY delay = 0 cycles

BREADY delay = 1 cycle

BREADY delay = 5 cycles

We're effectively testing every path through our control logic.

That is where the constrained-random + coverage portion of your project becomes meaningful.

# 21. The read side is much simpler

For comparison:

C0          C1          C2          C3

│           │           │           │

ARVALID     █████

ARREADY     █████

↑

AR

RVALID                  █████████

RREADY                  ─────████

↑

R

RDATA

RRESP

Internally:

READ_IDLE

│

│ AR handshake

▼

READ_RESPONSE

│

│ R handshake

▼

READ_IDLE

If RREADY is low:

READ_RESPONSE

│

└─────── stay here

until:

RVALID && RREADY

# 22. One thing I want you to notice

There are three different concepts here:

### 1. Channel handshake

VALID && READY

### 2. Transaction completion

For a write:

AW received

+

W received

+

B response accepted

For a read:

AR received

+

R response accepted

### 3. Register operation

For a write:

AW + W received

↓

register update

These are not the same event.

That's an extremely important distinction in AXI verification.

# 23. Our final write-control mental model

Think of the slave as a small clerk handling one customer:

CUSTOMER

│

┌────────┴────────┐

│                 │

ADDRESS             DATA

│                 │

▼                 ▼

[store AW]         [store W]

│                 │

└────────┬────────┘

│

Both present?

│

YES

│

▼

UPDATE REGISTER

│

▼

ISSUE RESPONSE

│

▼

WAIT FOR BREADY

│

YES

│

▼

IDLE

That is essentially our entire write datapath.

FINAL ARCHITECHTURE
                     AXI4-Lite SLAVE

│

┌────────────────┴────────────────┐

│                                 │

WRITE                             READ

│                                 │

AW capture                         AR capture

│                                 │

W capture                           RD_DEC

│                                 │

WR_EXEC                          RDATA/RRESP

│                                 │

Address decode                       RVALID

│                                 │

RW / invalid                    RREADY

/      \

update   SLVERR

│

COUNT

│

BVALID

ISSUES
Issue 1: Vivado Hierarchy / Interface Visibility

Problem: Vivado displays duplicate tb_top.sv entries and fails to show the axi4lite_if instance under tb_top.

Cause: UVM class files are added as standalone SystemVerilog sources while also being included in tb_top.sv, causing recursive hierarchy parsing.

Solution:

- Change all 9 UVM class files to Verilog Header in Source Node Properties.

- Ensure tb_top.sv does not include itself or axi4lite_if.sv.

- Refresh the hierarchy.

Note: The missing interface node may be a Vivado GUI limitation. Successful compilation and simulation are the definitive checks that the interface is correctly connected.

## Issue 2: UVM Simulation Hanging at Time 0

Problem: Compilation and elaboration succeed, and the UVM driver starts a WRITE transaction, but the simulation hangs without monitor, scoreboard, or completion messages.

Cause: The driver starts at time 0 while the active-low reset (resetn) is still asserted. It sees READY signals high during reset, incorrectly assumes the handshake is complete, and waits indefinitely for a BVALID response.

Solution: In axi4lite_driver.sv, make the driver wait for reset release before starting transactions:

// Wait for system reset to release before driving stimulus

wait(vif.aresetn === 1'b1);

@(posedge vif.aclk);

Place this before the driver's forever loop in run_phase.

Expected result: The driver begins transactions only after reset is released, allowing valid AXI4-Lite handshakes and preventing the simulation from hanging at startup.

Issue-3
The error occurs because Vivado is using UVM 1.2, where the API for the uvm_report_server changed. In UVM 1.2, the compose_report_message function requires two arguments (the message object and a string for the filename), whereas UVM 1.1d only required one.

To resolve this, update the function signature in your custom report server to accept the second string argument and pass it along to the super call.

Vivado’s SystemVerilog compiler is extremely strict about virtual method signatures matching the base class exactly, right down to the specific argument names used in the UVM 1.2 source code.

In UVM 1.2, the second argument isn't just any string; it is specifically named report_object_name. If the names don't match, Vivado throws an inheritance error.

Replace your entire axi4lite_test.sv with this updated code that uses the exact UVM 1.2 signature for the report server:

`timescale 1ns / 1ps

import uvm_pkg::*;

`include "uvm_macros.svh"

// Custom report server to strip the UVM boilerplate from messages tagged "CLEAN"

class clean_report_server extends uvm_default_report_server;

// EXACT UVM 1.2 signature match required by Vivado

virtual function string compose_report_message(uvm_report_message rm, string report_object_name = "");

if (rm.get_id() == "CLEAN") begin

return rm.get_message(); // Return purely the raw string

end else begin

return super.compose_report_message(rm, report_object_name); // Retain standard formatting

end

endfunction

endclass

class axi4lite_test extends uvm_test;

`uvm_component_utils(axi4lite_test)

axi4lite_env env;

virtual axi4lite_if.master  vif_master;

virtual axi4lite_if.monitor vif_monitor;

function new(string name = "axi4lite_test", uvm_component parent = null);

super.new(name, parent);

endfunction

virtual function void build_phase(uvm_phase phase);

clean_report_server srv;

super.build_phase(phase);

// Apply the custom report server

srv = new();

uvm_report_server::set_server(srv);

env = axi4lite_env::type_id::create("env", this);

if (!uvm_config_db#(virtual axi4lite_if.master)::get(this, "", "vif_master", vif_master))

`uvm_fatal(get_type_name(), "Master virtual interface not found in config_db")

uvm_config_db#(virtual axi4lite_if.master)::set(this, "env.agent.driver", "vif", vif_master);

if (!uvm_config_db#(virtual axi4lite_if.monitor)::get(this, "", "vif_monitor", vif_monitor))

`uvm_fatal(get_type_name(), "Monitor virtual interface not found in config_db")

uvm_config_db#(virtual axi4lite_if.monitor)::set(this, "env.agent.monitor", "vif", vif_monitor);

endfunction

virtual task run_phase(uvm_phase phase);

axi4lite_sanity_seq seq;

phase.raise_objection(this);

`uvm_info("CLEAN", "============================================================\n              AXI4-LITE UVM VERIFICATION\n============================================================\n\n[TEST] Starting sanity test\n", UVM_LOW)

seq = axi4lite_sanity_seq::type_id::create("seq");

seq.start(env.agent.sequencer);

phase.drop_objection(this);

endtask

// Report Phase: Print the clean summary

virtual function void report_phase(uvm_phase phase);

uvm_report_server srv = uvm_report_server::get_server();

int err_cnt  = srv.get_severity_count(UVM_ERROR) + srv.get_severity_count(UVM_FATAL);

int warn_cnt = srv.get_severity_count(UVM_WARNING);

string res   = (err_cnt == 0) ? "PASS" : "FAIL";

`uvm_info("CLEAN", $sformatf("\n------------------------------------------------------------\nRESULT: %s\nERRORS: %0d\nWARNINGS: %0d\n------------------------------------------------------------\n", res, err_cnt, warn_cnt), UVM_LOW)

endfunction

endclass

Phase 5 — Functional coverage
Add coverage for:

- READ/WRITE

- register/address categories

- WSTRB

- AW/W ordering

- backpressure
Then identify and close coverage gaps.

Phase 6 — SVA assertions
Add protocol assertions for things like:

- VALID/READY handshakes

- BVALID stability when BREADY=0

- RVALID/RDATA/RRESP stability when RREADY=0

- other AXI protocol rules.

Final regression
Run everything together and produce the final verification results.